| Event: Christchurch Earthquake |  | Section/Function/agency: Science Liaison Team |
| --- | --- | --- |
| Date: 01/03/2011 | Time: 1200 |  |
| Operational Period: |  | Originator Record #: 004 |

Current situation (incidents, action taken, resource status, personnel numbers etc)

Earthquake Activity:

The 22nd February M6.3 aftershock had about a 25% chance of occurring within one year starting on February 1st. The sequence of aftershocks generated by the M6.3 quake was unusually active in the first 24 hours with 39 aftershocks > M4, and 4 aftershocks >M5. Seismologists are currently working to understand the unusually high activity in the first day. Models predict that in the next week we should expect 1–10 aftershocks >M4 and within 3 weeks the number should drop to one M4 every 3 to 4 days. Given that the number of aftershocks prior to the magnitude 6.3 aftershock was slightly lower than average, it is likely that the number of aftershocks will trend toward the lower end of what is forecast.  This rate will then continue to decrease in a similar manner to what it was prior to the magnitude 6.3 aftershock.  After one year we expect roughly one M4 event per month.  A table of forecast events for the next year is shown below.  The average forecast number of events is given, with the possible range shown in parentheses.

| Forecast | In One Week | In 30 Days | In One Year |
| --- | --- | --- | --- |
| M4-4.9 | 5.1 (1-10) | 13 (6-21) | 44.5 (32-58) |
| M≥5 | 0.5 (0-2) | 1.2 (0-4) | 4.2 (1-9) |

A possibility remains for the occurrence of another aftershock >M6, however it is more likely that such an event will not occur.  As with smaller aftershocks, the chance of a larger aftershock decreases as we get farther away in time from the magnitude 7.1 earthquake.

See graphs attached - Appendix A illustrates the magnitude and frequency of aftershocks from 22 to 28 February 2011, and Appendix B illustrates the aftershock decay since the M7.1 earthquake of 4 September 2010.

Geology & Geotechnical: Daily geotechnical meetings are being held at 0800 at Opus, Moorhouse Ave. A landslide geodatabase is currently being set up within the Emergency Operations Centre GIS team.  Data collated in the field, from either rapid response assessments, reconnaissance or research data gathering will be entered into the database at the end of each day and made available to the 0800 geotechnical meetings and Emergency Operations Centre Ops.  The geodatabase will be supported by University of Canterbury students working offsite. Calls for immediate geotechnical assistance will be coordinated through the Science Liaison desk as the single point of contact.  Geotechnical engineers will then be tasked (either directly, or through the Opus earthquake call centre), results of assessments will be phoned back in and relayed to Emergency Operations Centre Ops each morning, and also entered into the geodatabase.

Liquefaction: Yesterday, 9 two-person T&T teams continued to move around suburbs making reconnaissance observations of the occurrence and severity of liquefaction and lateral spreading with respect to properties. They have also been concentrating on recording initial observations with respect to severe lateral spreading and liquefaction along stream and river sides.  To date all work remains observational, no testing or more detailed site investigations have commenced.  T&T staff have been developing GIS-based maps of observations recorded.  These can be made available, but will require approval for release from EQC.

The UC team, with assistance from visiting Japanese colleagues, continued with a systematic reconnaissance of the New Brighton, Southshore, North New Brighton, Parklands, Westhaven and Shirley areas yesterday.  Data is being captured on maps, but this is currently hand-drawn.

Outstanding Issues / Challenges / Problems:

Geology & Geotechnical: The number of enquiries re rock and slope instability coming directly to the Science Liaison Team is becoming unmanageable. A procedure needs to be developed to help manage the situation.

Liquefaction: Data capture and entry into a common GIS database platform is urgently required.  Currently there is no staff resource to undertake GIS entry as yet for the UC led team, and University buildings remain closed to staff, preventing any access to computing resources required.  As each day passes the detail of liquefaction evidence is being lost quickly with the clean up. The teams are endeavouring to mitigate the loss of evidence by continuing to focus on reconnaissance and photographic documentation.

Anticipated Priorities / Activities: (For future operational periods)

Earthquake Activity & Deformation: Seismologists are currently working to accurately locate the aftershocks and identify the causative faults. Analysis of satellite data continues, to determine areas of uplift and subsidence.

Engineering: Research projects may begin when appropriate.

Liquefaction: Developing the GIS capability to support the UC team is a high priority.  In conjunction with that we will also look into ways of coordinating data into a compatible platform with respect to other organisations so data can be effectively shared/combined (T&T; GNS Science; UC; LU; ECan).  UC will coordinate this with GNS science staff, and gain advice on a common best practice approach.  Until a contract staff member is in place, little progress is likely.  We will also require UC HR staff to quickly process a fixed-term staffing contract to engage the GIS support.

Geospatial: The need for LiDAR data has been reiterated, and planning for its acquisition is underway.

APPENDIX A:

Magnitude and frequency of aftershocks from 22 to 28 February 2011.

APPENDIX B:

Aftershocks decay since M7.1 earthquake of 4 September 2010.

A plot showing the daily recorded number of aftershocks >M4, shown by yellow dots, compared with the forecast number of events.  The black line shows the daily forecast number of events with 95% confidence bounds (the expected range) given by the grey lines.  The forecasts are calculated using a model based on the modified Omori Law.