| Event: Christchurch Earthquake |  | Section/Function/agency: Science Liaison Team |
| --- | --- | --- |
| Date: 03/03/2011 | Time: 1200 |  |
| Operational Period: |  | Originator Record #: 006 |

Current situation (incidents, action taken, resource status, personnel numbers etc)

Earthquake Activity & Deformation: Appendix A illustrates the magnitude and frequency of aftershocks from 22 February to 3 March 2011.

Geology & Geotechnical:

- Daily 0800 meeting of geotechnical engineers continues at Opus.  Most areas of immediate concern have now been assessed and residents evacuated if necessary.  A risk management process is now being developed to systematically assess rockfall and landslide hazards along the Port Hills.  The hills have been divided up into geographical areas and geotechnical teams have been assigned to each area.  Information from the teams is being fed into the Landslide Geodatabase, administered from the GIS section of the Emergency Operations Centre.  The mapping is also being coordinated with Tonkin & Taylor for their EQC assessments.

- The risk management approach is important to avoid unnecessary evacuations and to quickly assess those areas where people have been evacuated, so that people can be allowed back to their homes as soon as possible.   This process needs to be coordinated in the Emergency Operations Centre.

- Ops continues to receive requests from Operation Suburb for geotechnical assistance, and from members of the public concerned about rockfall, and these are being assigned to the geotech teams already out in the field.

- Ground party damage assessment: GNS landslide response team, along with UC and local consultants, continued assessing landslide damage using the proforma hazard assessment sheet. In the last 2 days more attention has been given to Cashmere / Huntsbury area (including looking for tectonic features on flats below) and Sumner/Redcliffs.

- Aerial reconnaissance: An immediate report from a reconnaissance flight on 26 February has been completed.

- Systematic mapping of landslide, rockfall and other slope-related ground damage commenced in Lower Hutt using the recently acquired aerial photography.

Monitoring:

- Monitoring: GeoNet staff have installed continuous GPS stations on top of 2 significant landslides (Kinsey Terrace, Glendavere), with addition of a weather station, upgraded by GeoNet with remote communications to transfer data out, while two sites on Balmoral Terrace remain on manual downloading, with processing in Lower Hutt.

- GNS staff are using a Terrestrial Laser Scanner at Redcliffs School to scan the rockfall sites below Glendevere and Balmoral.

- Several small accelerometers have been installed in locations spaced down Mt Hornibrook Rd.

Liquefaction: Both reconnaissance teams continued to cover areas of the city yesterday, and a new group has arrived from USA to support the UC team.

Geospatial & Databases: Set up and data entry into the centralised GIS-based database continued, for use by USAR and City Council for evacuation / life safety assessments.

Building Damage: Two GNS Science staff yesterday surveyed several suburbs across Christchurch to assess the nature, distribution and degree of structural damage following 22nd February earthquake. The suburbs visited include Merivale, Addington, Sydenham, Phillipstown, Linwood, Riccarton, Ferrymeade, Mt Pleasant, Redcliffs and Heathcote valley areas. They looked into residential, industrial and commercial (business) buildings. Preliminary and rapid assessments suggest the degree of damage for residential buildings across Christchurch is variable. On average, it may be about 20% minor damage, 70% moderate to severe damage and 10% could be complete damage. In suburbs close to the epicentre such as Mt Pleasant and Redcliffs, most of the houses were severely damaged due to ground shaking and landslide. Houses closer to the river beds were invariably damaged due to liquefaction induced ground displacement.

Outstanding Issues / Challenges / Problems:

Liquefaction: There is an urgent need to find a suitable room for the UC team to be able to meet at the end of each day and debrief, as well as plan the next days work.

Anticipated Priorities / Activities: (For future operational periods)

Landslides:

- Continuing reconnaissance inspections, including tectonic implications as allows.

- Providing advice to USAR teams on landslide related ground damage, including where requested, survey data relating to ground stability.

- Laser scanning in Redcliffs/Sumner will be completed (03 March) for this week.

- Systematic API mapping of landslide, rockfall and other slope-related ground damage.

Geospatial & Databases: Development of the landslide database will continue, to allow systematic assessment by recovery teams, with respect to landslide issues.

Building Damage: Today the focus will be on buildings in the CBD area.

APPENDIX A:

Magnitude and frequency of aftershocks from 22 February to 3 March 2011.