| Event: Christchurch Earthquake |  | Section/Function/agency: Science Liaison Team |
| --- | --- | --- |
| Date: 05/03/2011 | Time: 1200 |  |
| Operational Period: |  | Originator Record #: 008 |

- Current situation (incidents, action taken, resource status, personnel numbers etc)

- Geospatial:

- - High resolution aerial photos flown on 24 February are being used for liquefaction and landslide field work.

- - Planning for LiDAR acquisition flight continues.

Earthquake Activity & Deformation:

- Aftershocks continue to decrease in frequency.

- Landslides:

- Emphasis of investigations now shifting from area life safety to individual site assessment and monitoring.

- Investigation activities by a number of organisations/companies coordinated through daily meetings.  Landslide damage assessed using standard template and fed back to centralised landslide geodatabase at the Emergency Operations Centre.

- Continuous GPS monitoring continues on Kinsey Terrace, Glendavere and Balmoral Terrace.

- Mapping of landslides and rockfalls being mapped from aerial photos by GNS Science in Wellington.

- Mark Yetton of Geotech Consulting Ltd attended a meeting held by Ruth Dyson at Redcliffs Supermarket yesterday.  He explained the situation to people there and said the response from the community was positive.

- Liquefaction:

- University of Canterbury gained access to CBD on Friday 04 March and spent much of the day examining evidence for liquefaction and lateral spreading between Cambridge Terrace and Kilmore Street.  The Japanese geotechnical team continued with investigations adjacent to the Heathcote River.  The US GEER ( ) team visited various selected localities in the eastern suburbs – New Brighton/Bexley.

- The Tonkin & Taylor team have completed a first stage suburb by suburb reconnaissance of liquefaction occurrence.  They are now in the second stage street by street assessment of the worst affected areas.  The Tonkin & Taylor team are not working over this weekend (5/6 March).

Buildings/strucutural engineering:

- A meeting has been held at NZTA, involving people from GNS, University of Canterbury, Opus and international specialists to discuss the nature and degree of damage for large highway bridges.

- GNS Science researchers have been examining structural damage to buildings.

- International researchers are arriving in the coming weeks to inspect and collect data on building damage.

Socio-economic research:

- No activity at this stage.

- Outstanding Issues / Challenges / Problems:

Landslides:

- Communication with community has been poor.  Rectifying this over the coming days with information on website, tweets, notice boards and at community briefings.

Liquefaction:

- University of Canterbury team is hoping to employ GIS expertise to support the development of a geospatial database for liquefaction, and to support liaison between the various organisations involved with liquefaction mapping and site investigation.

- Anticipated Priorities / Activities: (For future operational periods)

- Geospatial:

- A LiDAR acquisition flight is being organised for the coming days.  Areas of interest are being compiled from a number of organisations.

- Landslides:

- Reconnaissance inspections continue, providing advice to USAR teams on landslide related ground damage, including where requested, survey data relating to ground stability.

- Systematic Aerial Photo Interpretation mapping of landslide, rockfall and other slope-related ground damage.

- Development of the landslide geodatabase and associated maps will continue.

- Community meetings are planned for the hill suburbs on Monday 07 March 2011.  It is intended that a geotech professional attend these meetings to explain rockfall and landslide issues.

- A plan for systematic assessment of rockfall hazards in evacuated areas and levels of acceptable risk is being developed today and tomorrow in order to get people back into their homes as quickly as possible where it is reasonable to do so.  This must be communicated to residents in the hill suburbs.

- Liquefaction:

- University of Canterbury coordinated Japanese and US GEER (Geotechnical Extreme Event Reconnaissance) teams will be departing over the weekend.  Over the coming days discussions will focus on more detailed investigation of critical areas affected by liquefaction and lateral spreading.  The University of Canterbury intend to develop a liquefaction geodatabase.

- Once Tonkin & Taylor have finished their second stage street by street assessment of liquefaction they will move into a property by property land damage assessment for EQC purposes.

Buildings/structural engineering:

- Several international researchers will be arriving in Christchurch over the coming weeks to inspect and collect data on buildings.  These visits are being coordinated through the University of Canterbury and the New Zealand Society for Earthquake Engineering.