| Event: Christchurch Earthquake |  | Section/Function/agency: Science Liaison Team |
| --- | --- | --- |
| Date: 06/03/2011 | Time: 1200 |  |
| Operational Period: |  | Originator Record #: 009 |

- Current situation (incidents, action taken, resource status, personnel numbers etc)

- Geospatial:

- - High resolution aerial photos flown on 24 February are being used for liquefaction and landslide field work.

- - Planning for LiDAR acquisition flight continues.

Earthquake Activity & Deformation:

- Aftershocks continue to decrease in frequency.

- Agnes Reyes and Bruce Christenson (GNS) will start hot spring work today. They will be testing springs at Cass, Rapaki, Motukarara, Purau, Heathcote and Ferrymead.

- Landslides:

- See attached Figure 1 for updated landslide geodatabase.

- There were no call outs for geotechnical engineers or the USAR geotechs overnight.  There had been some concern with the rainfall overnight potentially causing some movement of landslides, and the community was asked in a media release to be viligant and report any movement or widening of tension cracks.

- Emphasis of investigations now shifting from area life safety to individual site assessment and monitoring.

- Teams of geotechnical engineers are now prioritising more detailed assessments for evacuated areas.

- Grant Dellow from USAR is systematically working through the evacuated areas over the next two days to ensure that houses that should be evacuated are red stickered, to remove confusion.

- Continuous GPS monitoring continues on Kinsey Terrace, Glendavere and Balmoral Terrace.

- Mapping of landslides and rockfalls being mapped from aerial photos by GNS Science in Wellington.

- Team of rockslope engineers (Geovert) have been working to secure loose and unstable rocks on the hillslopes above Bridle Path Road to keep the heavy vehicle route to Sumner open.  They will then be moving to work along Wakefield Ave in Sumner area over the coming days.

- Ops water and waste have been supplied with a map (attached Figure 2) showing areas of landsliding on the Port Hills that may be prone to further instability if water services are restored and there are water leaks.

- Liquefaction:

- The Tonkin & Taylor team have completed a first stage suburb by suburb reconnaissance of liquefaction occurrence.  They are now in the second stage street by street assessment of the worst affected areas.  The Tonkin & Taylor team are not working over this weekend (5/6 March).

Buildings/strucutural engineering:

- Nothing further to report today.

Socio-economic research:

- No activity at this stage.

A 1 page earthquake science factsheet was produced overnight and is available for the community briefings from today.  It will also be available on the www.canterburyearthquake.govt.nz website.

- Outstanding Issues / Challenges / Problems:

- Anticipated Priorities / Activities: (For future operational periods)

- Geospatial:

- A LiDAR acquisition flight is being organised for the coming days.  Areas of interest are being compiled from a number of organisations.  Lifelines coordinators would like the acquisition of LiDAR to be brought forward as there are concerns with levels around the Estuary and lower Avon/Heathcote with spring tides two weeks away.

- Landslides:

- Reconnaissance inspections continue, providing advice to USAR teams on landslide related ground damage, including where requested, survey data relating to ground stability.

- Systematic Aerial Photo Interpretation mapping of landslide, rockfall and other slope-related ground damage.

- Development of the landslide geodatabase and associated maps will continue.

- Community meetings are planned for the hill suburbs on Sunday 06 March and Monday 07 March 2011.  Grant Dellow (USAR) and Mark Yetton (Geotech Consulting Ltd) will attend these meetings to explain rockfall and landslide issues.

- Liquefaction:

- Over the coming days University of Canterbury research will focus on more detailed investigation of critical areas affected by liquefaction and lateral spreading.  The University of Canterbury intend to develop a liquefaction geodatabase.

- Once Tonkin & Taylor have finished their second stage street by street assessment of liquefaction they will move into a property by property land damage assessment for EQC purposes.

Buildings/structural engineering:

- Several international researchers will be arriving in Christchurch over the coming weeks to inspect and collect data on buildings.  These visits are being coordinated through the University of Canterbury and the New Zealand Society for Earthquake Engineering.

Figure 1: Landslide geodatabase as at 1000 Sunday 06 March 2011

Figure 2: Map supplied to Ops water and waste by the geotech GIS people on Sunday 06 March 2011